# Network Layer Security – IPSec

## Introduction to IPSec

1. What is IPSec?

IP Security (IPSec) is a suite of protocols developed by the IETF to secure data at the network layer of the OSI model. Unlike HTTPS (application layer) or TLS (transport layer), IPSec protects traffic at the IP packet level. This means every packet that leaves one machine and travels over the Internet can be:

- Encrypted (confidentiality)

- Authenticated (verifying sender)

- Integrity-protected (detecting changes in transit)

- Replay-protected (blocking reused packets)

So even if an attacker intercepts packets, they won’t be able to read, alter, or replay them without being detected.

2. IPSec Protocol Components

- Authentication Header (AH)

  - Ensures integrity and authenticity of packets.

  - Provides protection against replay attacks.

  - Does not encrypt the payload (no confidentiality).

- Encapsulating Security Payload (ESP)

  - Provides confidentiality by encrypting the payload.

  - Also ensures integrity and authentication.

  - Most commonly used IPSec protocol in practice.

- Security Associations (SA)

  - Defines the relationship between two communicating parties.

  - Includes algorithms, keys, and lifetime of the secure connection.

- Key Management

  - IPSec uses the Internet Key Exchange (IKE) protocol to negotiate and exchange cryptographic keys dynamically.

Great question! Let’s break it down clearly for you and your students.

### Internet Key Exchange (IKE)

1. What is IKE?

Internet Key Exchange (IKE) is a protocol used within IPSec to establish Security Associations (SAs) and to negotiate cryptographic keys between two devices.

In simple terms:

- IPSec provides encryption and authentication at the IP layer.

- But before IPSec can protect packets, both sides must agree on:

  - Which encryption algorithm to use (AES, 3DES, etc.)

  - Which authentication method (hashes, digital signatures, etc.)

  - Keys to use for encryption/decryption

- IKE is the “handshake” protocol that sets all of this up automatically and securely.

2. Where does IKE fit in IPSec?

IPSec itself doesn’t define how keys are exchanged. IKE fills this gap.

- It runs on UDP port 500.

- It uses the ISAKMP framework (Internet Security Association and Key Management Protocol).

3. Versions of IKE

- IKEv1

  - Original version (RFC 2409).

  - Works in two phases:

    - Phase 1: Establishes a secure channel (ISAKMP SA).

    - Phase 2: Negotiates IPSec SAs (encryption/authentication for data).

  - Known to be slower and more complex.

- IKEv2

  - Improved version (RFC 7296).

  - Faster, more secure, and simpler.

  - Handles mobility (changing IPs, e.g., smartphones switching from Wi-Fi to 4G).

  - Provides built-in DoS resistance.

4. How IKE Works

Step 1: SA Negotiation

- Two peers (e.g., VPN gateways) agree on algorithms and methods (AES, SHA, DH group, etc.).

Step 2: Key Exchange

- Uses Diffie-Hellman (DH) to establish a shared secret key.

- Keys are generated without directly sending them across the network.

Step 3: Authentication

- Each side proves identity using:

  - Pre-shared keys (PSK), or

  - Digital certificates, or

  - Public/private key signatures.

Step 4: IPSec Setup

- Once keys and methods are agreed upon, IPSec SAs are created.

- IPSec then starts protecting actual traffic.

5. Why is IKE Important?

Without IKE, network administrators would have to manually configure keys for every IPSec connection. This would be:

- Impractical (imagine thousands of connections).

- Insecure (static keys are vulnerable).

IKE makes IPSec scalable, secure, and automatic.

3. Why IPSec is Important

- Secures VPN connections between organizations.

- Protects corporate networks when employees connect remotely.

- Provides a standard method for encrypting IP traffic across the Internet.

- Supports IPv4 and IPv6.

## IPSec Modes: Transport Mode vs Tunnel Mode

1. Concept Overview

IPSec can work in two modes:

- Transport Mode:

  - Protects the payload (the data from the transport layer, e.g., TCP, UDP).

  - The original IP header is left intact.

  - Used for end-to-end communication between two hosts.

  - Routers in the middle can still see the source/destination addresses.

- Tunnel Mode:

  - Protects the entire original IP packet (header + payload).

  - A new outer IP header is added for routing.

  - Commonly used for gateway-to-gateway or gateway-to-client VPNs.

  - Hides the internal network addressing scheme from outsiders.

### 2. Transport Mode

How It Works:

- The packet’s IP header is not encrypted.

- Only the transport-layer payload (e.g., TCP segment or UDP datagram) is protected.

- Useful when two endpoints already know each other (host-to-host).

Example Scenario:
Two servers within the same company communicating securely over the LAN.

Security Properties:

- Confidentiality: Yes (if ESP is used).

- Integrity: Yes.

- Address hiding: No (IP headers remain visible).

### 3. Tunnel Mode

How It Works:

- The entire original IP packet (including header) is encrypted and encapsulated.

- A new IP header is added with the VPN gateway addresses.

- Outsiders only see communication between gateways, not between the original hosts.

Example Scenario:
A branch office VPN router connects to headquarters VPN router. Inside traffic between employee PCs is hidden.

Security Properties:

- Confidentiality: Yes.

- Integrity: Yes.

- Address hiding: Yes (original IP addresses are invisible).

4. Visual Comparison

Transport Mode (host-to-host)

+-------------------------+

| IP Header | ESP | Data |

+-------------------------+

Tunnel Mode (gateway-to-gateway)

+--------------------------------------------+

| New IP Header | ESP | Original IP + Data   |

+--------------------------------------------+

## Virtual Private Networks (VPNs)

1. What is a VPN?

A Virtual Private Network (VPN) is a technology that creates a secure, private communication channel over a public network (like the Internet).
Instead of leasing dedicated physical lines between sites, organizations use VPNs to tunnel their private traffic through the shared Internet.

Key Properties of VPNs:

- Provides confidentiality by encrypting traffic.

- Ensures integrity and authentication between sites or users.

- Supports remote access and site-to-site communication.

- Uses technologies like IPSec or SSL/TLS to secure communication.

2. Why Use a VPN?

- Remote Workers: Employees can securely connect to the office network from home.

- Branch Offices: Connect branch offices securely to headquarters without expensive leased lines.

- Privacy: Hides internal addressing from external observers.

- Cost-Efficiency: Uses the Internet instead of costly dedicated circuits.

3. Types of VPNs

3.1 Remote Access VPN

- Connects individual users to an organization’s private network.

- Example: An employee connecting securely from home to the office file server.

3.2 Site-to-Site VPN

- Connects entire networks together (gateway-to-gateway).

- Example: Two company branches sharing resources via the Internet.

4. How VPNs Work

- Encapsulation: The original private packet is wrapped inside a new packet.

- Encryption: The payload is encrypted to ensure confidentiality.

- Tunneling Protocols: IPSec, GRE, L2TP, OpenVPN, WireGuard, etc.

- Authentication: Uses pre-shared keys, digital certificates, or usernames/passwords.

5. Addressing in VPNs

- Inside the VPN, organizations use private IP addresses (RFC 1918 ranges: 10.0.0.0/8, 172.16.0.0/12, 192.168.0.0/16).

- Outside (on the public Internet), packets use public IP addresses.

- The VPN gateway translates between private and public addressing while encrypting/decrypting traffic.

## Tunneling

1. Concept of Tunneling

Tunneling is the process of encapsulating one type of network packet inside another.

- The inner packet: the original data (often using private IP addresses).

- The outer packet: provides routing information and often encryption.

When a packet is tunneled, intermediate routers on the Internet only see the outer header, while the actual data and addressing inside remain hidden and protected.

2. Why Tunneling?

- Allows private traffic to travel over a public network.

- Hides the original source and destination addresses.

- Enables use of private IP addresses that are not routable on the Internet.

- Supports VPN connections between different sites or remote clients.

Analogy:

- Post office only sees the outer envelope (destination = VPN gateway).

- Only when it reaches the intended gateway is the inner letter opened and delivered.

3. Types of Tunneling Protocols

Several tunneling protocols exist, including:

- IPSec Tunnel Mode (most common in corporate VPNs).

- GRE (Generic Routing Encapsulation).

- L2TP (Layer 2 Tunneling Protocol) often combined with IPSec.

- OpenVPN and WireGuard (modern tunneling solutions).

4. How Tunneling Works (Step-by-Step)

- A host inside a private network sends a packet (e.g., from 10.0.0.5 → 10.1.0.7).

- The VPN gateway encapsulates this packet:

  - The original packet (with private IPs) is placed inside a new packet.

  - A new IP header is added with the public IPs of the two gateways.

- The encapsulated packet travels across the Internet.

- At the receiving gateway, the outer header is removed (decapsulation).

- The original private packet is delivered to the destination host.

5. Visual Example

Without tunneling:

[10.0.0.5] ----> [Internet Router] ----> [10.1.0.7]   (not possible: private IPs not routable)

With tunneling:

[10.0.0.5] ---> [Gateway A] ---[Encrypted Tunnel]---> [Gateway B] ---> [10.1.0.7]

Outer header (seen by Internet): PublicIP_A → PublicIP_B
Inner header (hidden inside tunnel): 10.0.0.5 → 10.1.0.7

## Addressing in a VPN

1. Concept of Addressing in VPNs

A Virtual Private Network (VPN) allows private networks to communicate securely across the Internet. Since organizations usually rely on private IP addresses (defined in [RFC 1918]), VPNs must translate and encapsulate these private addresses so they can traverse the public Internet.

Private Address Ranges (RFC 1918):

- 10.0.0.0 – 10.255.255.255

- 172.16.0.0 – 172.31.255.255

- 192.168.0.0 – 192.168.255.255

These addresses are not routable on the public Internet. Without VPNs or NAT, they cannot communicate across different networks.

2. Addressing Challenge

- Inside the organization: hosts use private IP addresses.

- On the Internet: routers expect public IP addresses.

- VPN solves this by:

  - Encapsulating packets with a new outer IP header containing public IP addresses of the gateways.

  - Ensuring that private IP traffic remains hidden and secure.

3. Addressing in VPN Modes

Transport Mode

- The original IP header is visible.

- Private IPs can only be used if the hosts are on the same private network.

- Less common for VPNs across the Internet.

Tunnel Mode

- The entire private IP packet (including header) is encapsulated inside another IP packet with public addresses.

- This is the most common mode for VPNs because it hides internal addressing from the outside world.

4. Example Scenario

Company A has a private subnet 10.0.0.0/24 at its HQ.
Company B has a private subnet 10.1.0.0/24 at its branch office.

- HQ Gateway Public IP: 203.0.113.1

- Branch Gateway Public IP: 198.51.100.1

When a host in HQ (10.0.0.10) wants to communicate with a host in the branch (10.1.0.5):

- Original packet:

Source: 10.0.0.10 → Destination: 10.1.0.5

- VPN Gateway encapsulates it:

Outer Header: Source 203.0.113.1 → Destination 198.51.100.1

Inner Packet: Source 10.0.0.10 → Destination 10.1.0.5

- Packet travels securely over the Internet.

- Branch Gateway decapsulates and delivers the original packet to 10.1.0.5.

Observation: The Internet only sees 203.0.113.1 ↔ 198.51.100.1, while internal IPs remain hidden.

Exercise 3: NAT + VPN

- Configure NAT on the VPN gateway.

sudo iptables -t nat -A POSTROUTING -s 10.0.0.0/24 -o eth0 -j MASQUERADE

- Without VPN, NAT translates private → public, but packets remain visible in plaintext.

- With VPN tunnel active, NAT + IPSec encapsulation ensures both:

  - Traffic is routable on the Internet.

  - Payload remains encrypted and private.

## Network Address Translation (NAT)

1. Concept of NAT

Network Address Translation (NAT) is a technique used in computer networks to map private IP addresses inside a local network to a public IP address (or a few public IPs) before sending traffic to the Internet.

Because IPv4 addresses are limited, NAT allows many devices in a private network to share a single public IP for external communication.

2. Why NAT?

- Address conservation: Private IP ranges (RFC 1918) are reused in millions of networks worldwide. NAT makes this possible.

- Security by obscurity: Internal hosts are hidden from the public Internet.

- Network flexibility: Organizations can change internal addressing without affecting external communication.

3. Types of NAT

3.1 Static NAT

- One private IP ↔ one public IP (1:1 mapping).

- Example: Exposing an internal web server with a fixed external address.

3.2 Dynamic NAT

- A pool of public IPs is shared among private hosts.

- Each internal host gets a temporary mapping to one of the available public IPs.

3.3 PAT (Port Address Translation) or NAT Overload

- Many internal hosts share a single public IP.

- Differentiation is made using source port numbers.

- This is the most common form of NAT in home routers.

4. NAT Workflow Example

Host: 10.0.0.10 (private) → wants to access a web server on the Internet.

- Internal packet before NAT:

Source: 10.0.0.10 → Destination: 142.250.183.174 (www.google.com)

- NAT router translates:

Source: 203.0.113.5 (public IP of router) → Destination: 142.250.183.174

and records the mapping in a translation table.

- Response from Google goes back to 203.0.113.5.

- NAT router consults its table and forwards it back to 10.0.0.10.

5. Advantages of NAT

- Saves IPv4 addresses.

- Provides basic protection by hiding internal topology.

- Allows seamless Internet access for many devices.

6. Limitations of NAT

- Breaks end-to-end connectivity (problematic for peer-to-peer apps, VoIP, gaming).

- Protocols embedding IP addresses in payloads (like FTP, SIP) may fail without NAT helpers.

- Adds processing overhead on routers.

Simple Exercises – One Machine Setup

1. Observe Private vs Public IPs (Addressing in VPNs)

- Open a terminal and run:

- ip addr

- Students will see:

  - A private IP (e.g., 192.168.x.x or 10.x.x.x) for the LAN interface.

  - A public IP if connected directly to the Internet.

Discussion: Why can’t 192.168.x.x be routed over the Internet?

2. Simulate Tunneling with Ping + Wireshark

- Install Wireshark on the same machine.

- Run in terminal:

- ping -c 4 www.google.com

- Capture packets in Wireshark.

you will observe:

- Source = their private IP.

- Destination = Google’s public IP.

Discussion: In a real VPN tunnel, this would be encapsulated inside another packet.

3. NAT in Action (with Internet Sharing)

On the same machine, run:

curl ifconfig.me

- This shows your public IP (what the Internet sees).

- Compare with your private IP (ip addr).

Observation: NAT is already working (your router translates your private IP → public IP).

4. Simulate a VPN Tunnel (SSH Local Port Forwarding)

Even on one machine, you can create a "mini tunnel" using SSH.
Run:

ssh -L 8080:google.com:80 user@localhost

- Then open browser: http://localhost:8080

- It forwards through an encrypted SSH tunnel, simulating VPN tunneling.

Observation: The packet to localhost is re-tunneled by SSH to Google.

## MCQs – IPSec & Network Layer Security

Q1.

Which of the following best explains why IPSec is considered a network-layer security protocol?
A) It secures only application data like HTTPS.
B) It operates between IP and transport layers, securing every packet.
C) It requires user authentication for each session.
D) It encrypts passwords at the application layer.
Answer: B

Q2.

In IPSec transport mode, which part of the packet remains unencrypted?
A) TCP/UDP payload
B) Entire IP packet
C) Original IP header
D) Encapsulated Security Payload (ESP) trailer
Answer: C

Q3.

Which IPSec mode is most suitable for a site-to-site VPN across the Internet?
A) Transport mode
B) Tunnel mode
C) Authentication Header only
D) SSL mode
Answer: B

Q4.

What is the main security risk if IPSec is used only in transport mode between two hosts?
A) Payload remains in plaintext.
B) IP headers are visible to attackers.
C) NAT cannot be used.
D) It consumes more bandwidth.
Answer: B

Q5.

Which of the following is not an advantage of VPNs?
A) Secure intra-organization communication.
B) Elimination of encryption overhead.
C) Ability to use the Internet instead of leased lines.
D) Privacy through tunneling.
Answer: B

Q6.

Tunneling works by:
A) Encrypting IP headers only.
B) Wrapping private packets inside new packets with public headers.
C) Using MAC addresses instead of IP addresses.
D) Sending traffic only within private networks.
Answer: B

Q7.

When a VPN encapsulates traffic:
A) The Internet sees only public gateway addresses.
B) Both private and public addresses are exposed.
C) Payloads remain unprotected.
D) NAT is not required.
Answer: A

Q8.

Which addressing issue is solved by VPN tunneling?
A) Limited number of TCP ports.
B) Non-routability of private IP addresses across the Internet.
C) Authentication of public IPs.
D) Latency introduced by routers.
Answer: B

Q9.

What is the biggest limitation of static NAT compared to PAT?
A) It requires one public IP per internal host.
B) It prevents encryption of payloads.
C) It cannot handle TCP connections.
D) It is slower than PAT.
Answer: A

Q10.

Which type of NAT is most common in home networks?
A) Static NAT
B) Dynamic NAT
C) PAT (Port Address Translation)
D) Bi-directional NAT
Answer: C

Q11.

Why is NAT often considered a security feature, even though it’s not true encryption?
A) It increases key length for encryption.
B) It hides internal IP addresses from the outside world.
C) It prevents packet sniffing.
D) It verifies sender identity.
Answer: B

Q12.

In a VPN using tunnel mode, which statement is correct?
A) The original IP header is hidden inside the encrypted packet.
B) Routers on the Internet must decrypt packets to forward them.
C) Only the transport-layer payload is encrypted.
D) Private IPs can be directly routed on the Internet.
Answer: A

Q13.

Which of the following could break IPSec if combined with NAT incorrectly?
A) ICMP Echo Requests
B) ESP packets without ports
C) DNS lookups
D) ARP requests
Answer: B

Q14.

What is the main difference between AH (Authentication Header) and ESP (Encapsulating Security Payload) in IPSec?
A) AH provides encryption, ESP does not.
B) ESP provides both encryption and authentication, AH does not encrypt.
C) Both AH and ESP provide encryption.
D) AH hides IP addresses, ESP does not.
Answer: B

Q15.

Which of the following best describes a site-to-site VPN?
A) A VPN between a user’s device and a corporate server.
B) A VPN connecting two different corporate networks over the Internet.
C) A VPN between two applications running on the same host.
D) A VPN used only for email encryption.
Answer: B

Q16.

Why can’t private IP addresses (e.g., 10.0.0.1) be routed across the Internet?
A) They are reserved by NAT routers for security.
B) They are not globally unique and could conflict with other networks.
C) They are encrypted by default.
D) They lack valid MAC addresses.
Answer: B

Q17.

Which is the biggest drawback of NAT for peer-to-peer applications?
A) NAT increases encryption overhead.
B) NAT hides public IP addresses, making direct connections difficult.
C) NAT requires more bandwidth.
D) NAT cannot be used in tunnel mode.
Answer: B

Q18.

Which statement about VPN tunneling is false?
A) Tunneling hides private addresses from the Internet.
B) Encapsulation creates an outer IP header.
C) Payload remains readable by intermediate routers.
D) VPN gateways perform encapsulation and decapsulation.
Answer: C

Q19.

Which of the following is a valid real-world use case of NAT with VPN?
A) Allowing multiple branch offices using private addresses to connect to HQ through one public IP.
B) Encrypting login credentials in web applications.
C) Blocking replay attacks by attackers.
D) Reducing bandwidth usage by compressing packets.
Answer: A

Q20.

If IPSec tunnel mode is enabled between two gateways, what does Wireshark show when capturing traffic in between?
A) Plaintext payload with private IPs visible.
B) ESP packets with outer public IP headers.
C) Encrypted packets with original headers exposed.
D) Only MAC addresses without IP information.
Answer: B